Lab 7 - DC Motor Driving using Relays and Interrupts

Student: Faisal Alazemi  |  ID: 23086427

Part v - Structured UART Report and Motor Event Log

#include "mbed.h"

struct MotorEvent {

int    timeMs;

int    tempC;

int    gasPpm;

const char* source;   // "AUTO","MANUAL","ISR"

bool   motorOn;

};

class EventLogger {

static const int CAP = 10;

MotorEvent buf[CAP];

int head = 0, count = 0;

public:

void add(int ms, int T, int G,

const char* src, bool on) {

buf[head % CAP] = {ms, T, G, src, on};

head++;

if (count < CAP) count++;

}

void printLatest() const {

if (count == 0) return;

const MotorEvent& e = buf[(head-1)%CAP];

char b[200];

snprintf(b, sizeof(b),

"+---- MOTOR EVENT ----+\r\n"

"| Time: %dms\r\n"

"| Event: %s\r\n"

"| Source: %s\r\n"

"| Motor: %s\r\n"

"| Temp: %dC | Gas: %dppm\r\n"

"+---------------------+\r\n",

e.timeMs, "State change",

e.source, e.motorOn?"ON":"OFF",

e.tempC, e.gasPpm);

uart.write(b, strlen(b));

}

} logger;

void printStatusReport(bool on, MotorMode md,

int T, int G,

int onCnt, int offCnt) {

char buf[256];

snprintf(buf, sizeof(buf),

"Motor: %s | Mode: %s | Speed: 100%%\r\n"

"Temp: %dC | Gas: %dppm\r\n"

"ON Count: %d | OFF Count: %d\r\n",

on?"ON":"OFF", modeName(md),

T, G, onCnt, offCnt);

uart.write(buf, strlen(buf));

}