Lab 5 - Keypad Alarm Deactivation with Dynamic Thresholds

Student: Faisal Alazemi  |  ID: 23086427

Part iv - Circular Event Log (5 Most Recent Events)

#include "mbed.h"

struct AlarmEvent {

int   tempC;

int   gasPpm;

int   tempThresh;

int   gasThresh;

char  trigger;   // 'T' or 'G'

int   timeMs;

};

const int LOG_CAP = 5;

AlarmEvent eventLog[LOG_CAP];

int writeIdx = 0;

int totalLogged = 0;

Timer sysTimer;

void recordEvent(int T, int G, int tTh, int gTh, char trig) {

eventLog[writeIdx] = {

T, G, tTh, gTh, trig,

(int)(sysTimer.elapsed_time().count() / 1000)

};

writeIdx = (writeIdx + 1) % LOG_CAP;

if (totalLogged < LOG_CAP) totalLogged++;

}

// Print all logged events:

// for (int i = 0; i < totalLogged; i++) {

//   int idx = (writeIdx - totalLogged + i + LOG_CAP) % LOG_CAP;

//   print eventLog[idx]...

// }